Name: Muhammad Najeeb Subah	 	 	   Registration No: 2025-CE-91

Date:   06/05/2026 	 	 Grade and Signature:  …………………

CA Lab 10 (Procedure steps)

1. Components Used:

In Proteus, resistors of required values were selected from the library. A DC voltage source (10V) was used. A DC ammeter and DC voltmeter (multimeter) were used for measurements along with ground.

DC Source (10V).

DC Ammeter.
Resistor.
DC Voltmeter (Multimeter).

2. Placing Components:

All components were selected from the “Pick Devices” menu in Proteus. The DC source, resistors, ground, and measuring instruments were placed on the workspace.

3. Circuit Connections:

The circuit was connected as per the given diagram. The resistors were connected in the required network configuration, and the load resistance was connected across the output terminals.

4. Measurement of Current (Initial Setup):

The DC ammeter was connected in series with the load resistance. The circuit was completed, and current through the load was observed.

5. Determination of Thevenin Voltage (Vth):

To find Vth, the load resistance was removed from the circuit.
A voltmeter was connected across the open terminals, and the open circuit voltage was measured as 5V.

6. Determination of Thevenin Resistance (Rth):

To find Rth, the DC source was replaced with a short circuit.
A multimeter (ohmmeter mode) was connected across the open terminals, and the equivalent resistance was measured as 495Ω.

7. Equivalent Circuit Construction:

A new circuit was constructed using the obtained values:

- Thevenin Voltage = 5V

- Thevenin Resistance = 495Ω

The load resistance was connected in series with Rth and the DC source.

8. Running the Simulation:

After completing all connections, the simulation was started using the Run button in Proteus.

9. Observation and Verification:

The current through the load in the equivalent circuit was measured and compared with the original circuit.
It was verified that both currents are equal, thus confirming Thevenin’s Theorem.